# CLASP2 data user guide

# Content:

## Acknowledgement statement

## Pointing

### IRIS co-observation data

## Description and graphical examples of data levels

### Brief description of data levels available through the VSO (“which data do I want?”)

### Lower data levels for all cameras

### Higher data levels for slitjaw data

### Higher data levels for spectropolarimetric dat

### Getting data from the VSO

### Averaging and estimation of uncertainties

### List of references

### Point of contact

# Acknowledgement statement

All publications and presentations using CLASP2 data must include the following acknowledgement statement:

CLASP2 is an international partnership between NASA/MSFC, NAOJ, JAXA, IAC, and IAS; additional partners include ASCR, IRSOL, LMSAL, and the University of Oslo. The CLASP2 research was funded by ISAS/JAXA as a Small Mission-of-Opportunity Program, by JSPS KAKENHI Grant numbers JP25220703 and 16H03963, 2015 ISAS Grant for Promoting International Mission Collaboration, 2016 NAOJ Grant for Development Collaboration, NASA Award 16-HTIDS16_2-0027, and Spanish participation through ERC Advanced Grant agreement 742265; and French hardware participation was funded by CNES funds CLASP2-13616A and 13617A.

# Pointing

CLASP2 flew on 11 April 2019 and pointed at three different positions on the Sun: at disk center for about 15 seconds for polarization calibration, at a plage position for about 2 min 26 sec, and near the limb for the rest of the flight. Figure 1 shows the three pointings overplotted on an AIA 304 image during that time.

Figure 1: 	The CLASP2 pointings at disk center, plage, and near the limb. 
Times are in UTC.

2.1  IRIS co-observation data

The IRIS co-observation data for CLASP2 can be found at: 	https://iris.lmsal.com/search/

OBSID:	3683603440

Note that there is also context data available at different times. Search for “CLASP-2” in the “DESC” box.

# Description and graphical examples of data levels

CLASP2 observed in the wavelength region around Mg II h & k. The data was taken with three cameras, two cameras for spectropolarimetry (SP1 and SP2) and one slitjaw camera (SJ). 
Table 1 shows the data levels for each camera, the ones shaded in green are available through the VSO.

|  | SJ | SP1 | SP2 | SP (SP1 and SP2 combined) |
| --- | --- | --- | --- | --- |
| Level-0 | X | X | X | - |
| Level-0.5 | X | X | X | - |
| Level-1 | X | X | X | X |

Table 1: Data levels for each camera (green = available through the VSO)

### 3.1  Brief description of data levels available through the VSO (“which data do I want?”)

Spectropolarimetric data:

Level-0.5: 	highest cadence data (200 ms), basic correction steps applied, no demodulation, 
no co-registration, no coordinate information.

You probably want this data if...	you do not care about polarimetry or coordinates and need the highest possible cadence for the intensities.

Level-1: 	lower cadence data (3.2 s), all correction steps applied, demodulated, 
co-registered, coordinate information.

You probably want this data if...		you want Stokes parameters and/or coordinates.

Slitjaw data:

Level-1: 	600 ms cadence, all correction steps applied.

3.2  Lower data levels for all cameras

Level-0:

- SJ: 	600 ms cadence

- SP1/SP2: 	200 ms cadence

- unit: DN

- data in primary FITS HDU

- raw data

- timestamp correction

- adding information to the FITS headers:

  - TARGET:  “Dark”, “Moving”, “SunCenter”, “Plage”, “Limb”

  - TELESCOP, INSTRUME, WAVELNTH, etc.

Graphical examples of the data:

Disk center:    SJ                                                      SP1                                                                         SP2

Plage:              SJ                                                      SP1                                                                         SP2

Limb:               SJ                                                      SP1                                                                         SP2

Note: The vertical black line in the SP1/SP2 data is caused by dust obscuring part of the slit.

Level-0.5:

- SJ: 	600 ms cadence

- SP1/SP2: 	200 ms cadence

- unit: electrons

- data in primary FITS HDU

- correction for:

  - bias

  - dark

  - gain

  - frame transfer image smear

- darks and exposures during slew discarded (TARGET = “Dark” or “Moving”)

- manually selected images discarded

Graphical examples of the data:

Disk center:    SJ                                                      SP1                                                                         SP2

Plage:              SJ                                                      SP1                                                                         SP2

Limb:               SJ                                                      SP1                                                                         SP2

## 3.3  Higher data levels for slitjaw data

Level-1:

- 600 ms cadence

- unit: electrons

- data in primary FITS HDU

- flatfield correction

- cutting off non-active pixel regions

- image rotation (90 CW) and horizontal flip

- cross-correlation with AIA 304 data to add coordinates to the FITS headers

Graphical examples of the data:

Disk center	Plage 	Limb

## 3.4  Higher data levels for spectropolarimetric data

Level-1:

- 3.2 s cadence

- unit: electrons

- Stokes I/Q/U/V in FITS extension 1/2/3/4

- polarization demodulation

- cutting off non-active pixel regions

- SP1: image rotation (90 CW)

- SP2: image rotation (90 CCW) and horizontal flip

- affine transformation:

  - shift (co-registration, slit drift compensation)

  - rotation (wavelength axis parallel to x-axis, slit axis parallel to y-axis)

- combining SP1 and SP2 to SP

  - compensation for (slightly) different SP2 spatial scale

  - process of combining SP1 and SP2 is described below

Graphical examples of the data:

Disk center: 	  SP1	  SP2

I                        Q                      U                       V                           I                        Q                      U                       V

SP (SP1 and SP2 combined)

I                      Q                     U                      V

Plage: 	  SP1	  SP2

I                        Q                      U                       V                           I                        Q                      U                       V

SP (SP1 and SP2 combined)

I                      Q                     U                      V

Limb: 	  SP1	  SP2

I                        Q                      U                       V                           I                        Q                      U                       V

SP (SP1 and SP2 combined)

I                      Q                     U                      V

How SP1 and SP2 are combined (description is for Stokes Q, same for Stokes U and V):

- Derive the fractional polarization at each spatial and wavelength pixel at each PMU rotation in each channel:
                                         	           qSP1=K1Q(x,λ,t)K1I(x,λ,t)=QSP1(x,λ,t)ISP1(x,λ,t)           qSP2=K2Q(x,λ,t)K2I(x,λ,t)=QSP2(x,λ,t)ISP2(x,λ,t)

where t is the PMU rotation number and K1 and K2 are the throughputs in SP1 and SP2

- Take the average of the fractional polarization between the two channels:                                                 qSP=qSP1(x,λ,t)+qSP2(x,λ,t)/2

Note: the averaged quantity qSP is the fractional polarization without any uncertainty 
in throughput. This process also cancels out I → Q/U crosstalk.

qSP=K1Q(x,λ,t)K1I(x,λ,t)+K2Q(x,λ,t)K2I(x,λ,t)/2=Q(x,λ,t)I(x,λ,t)

- Multiply the averaged fractional polarization with the sum of the SP1 & SP2 intensities:
QSP(x,λ,t)=qSP(x,λ,t)∙ISP1(x,λ,t)+ISP2(x,λ,t)=qSP(x,λ,t)∙ISP(x,λ,t)

- The final outputs are:      ISPx,λ,t,   QSPx,λ,t,   USPx,λ,t,   VSP(x,λ,t)
Advantage of this procedure:   ISPx,λ,t  directly provides the total number of e- collected by the two channels.

## 3.5  Getting data from the VSO

The CLASP2 data is available here: https://msfc.virtualsolar.org/CLASP2/

It is organized in zip files that contain all data levels for each instrument.

# Averaging and estimation of uncertainties

## Averaging

In order to achieve high S/N, it is highly recommended to temporally average/integrate the Stokes signals as follows:

- q=QSPx,λ,t1+QSPx,λ,t2+QSPx,λ,t3+QSPx,λ,t4+ ⋯ISPx,λ,t1+ISPx,λ,t2+ISPx,λ,t3+ISPx,λ,t4+ ⋯

- u=USPx,λ,t1+USPx,λ,t2+USPx,λ,t3+USPx,λ,t4+ ⋯ISPx,λ,t1+ISPx,λ,t2+ISPx,λ,t3+ISPx,λ,t4+ ⋯

- v=VSPx,λ,t1+VSPx,λ,t2+VSPx,λ,t3+VSPx,λ,t4+ ⋯ISPx,λ,t1+ISPx,λ,t2+ISPx,λ,t3+ISPx,λ,t4+ ⋯

Take the fraction after summation. Spatial and wavelength averaging can be done similarly.

## Uncertainties

The main source of uncertainties are the photon noise and readout noise (Ishikawa et al. 2014), and can be estimated as follows:

- ∆q=Δqphoton2+ΔqCCD2, ∆u=Δuphoton2+ΔuCCD2, ∆v=Δvphoton2+ΔvCCD2

- Δqphoton=Δuphoton=1aN,  Δvphoton=1cN

- ΔqCCD=ΔuCCD=nσsaN, ΔvCCD=nσscN

a=(1-cos(234°))/π,  c=-2sin(234°)/π : modulation coefficient (Song et al. 2022)

N= ISPx,λ,t1+ISPx,λ,t2+ISPx,λ,t3+ISPx,λ,t4+⋯ ISPx,λ,tM : total number of e-

n=2×16×nM : 	total number of accumulated exposures. ISPx,λ,ti consists of data from two 
channels and 16 exposures

nM: Number of summations (spatial/temporal/wavelength)

σs=6e-1 (read-out noise, Champey et al. 2014, 2015)

Note that the uncertainties caused by the read-out noise are usually negligible compared to the photon noise (Ishikawa et al. 2021, Rachmeler et al. 2022).

# List of references

### Instrument papers

Optical design and performance overview

- Song, D., et al. 2018, Proceedings of the SPIE, Vol. 10699 (alignment and performance verification of spectropolarimeter)

- Yoshida, M., et al. 2018, Proceedings of the SPIE, Vol. 10699 (alignment and performance verification of telescope)

- Tsuzuki, T., et al. 2020, Proceedings of the SPIE, Vol. 11444 (optical design of spectropolarimeter)

- Kubo, M., et al. 2016, ApJ, 832, 141 (slitjaw overview)

### Polarimetry (in addition to the list above)

- Song, D., et al. 2022, Solar Physics, Vol. 297 (polarization calibration)

- Ishikawa, R., et al. 2013, ApOpt, 52, 8205 (waveplate)

- Ishikawa, S., et al. 2015, SoPh, 290, 3081 (polarization modulation unit)

### Cameras

- Champey, P., et al. 2014, Proceedings of the SPIE, Vol. 9144

- Champey, P., et al. 2015, Proceedings of the SPIE, Vol. 9601

Science papers

- Ishikawa, R., et al. 2021, Science Advances, Vol. 7 (CLASP2 first results paper: WFA application)

- Rachmeler, L., et al. 2022, ApJ, Vol. 936 (CLASP2 first result paper: scattering polarization over the Mg II h & k lines)

- Li, H., et al. 2023, ApJ, Vol. 945, 144 (inversion application)

- Ishikawa, R., et al. 2023, ApJ, Vol. 945, 125 (detection of Hanle and MO effects)

- Afonso Delgado, D., et al. 2023, ApJ, Vol. 954, 218 (WFA application)

- Li, H., et al. 2024, ApJ, Vol. 975, 110 (Full-Stokes inversion)

### Reviews

- Trujillo Bueno, J. and del Pino Alemán, T. 2022, Annual Review of A&A, Vol. 60, 415-453

# Point of contact

David McKenzie

NASA Marshall Space Flight Center ST10

Huntsville, AL 35812

USA

david.e.mckenzie [at] nasa.gov